Specifications as stipulated by Reserve Bank of India for your

ready reference :-

Details      Thickness                    Minimum reinforcement

Walls        RCC (1:2:4)  30 cm    12mm dia Tor Steel at 6`` centre to centre both ways

Room (12``) Thick     and on both faces of the wall (a formation of

reinforcement matt(of about 6`` x 6``) on either face

of the wall  to be obtained

Floor         RCC(1:2:4) 30 cm         Same as in the case of walls

(12``) Thick

Roof         RCC (1:2:4) 30 cm         Same as in the case of Walls

You may please note the following points while adopting the above specifications :-

- Floor to be reinforced as indicated above where slabs are to be laid over the existing plain cement concrete flooring for vaults in ground floor and over existing RCC slabs in vaults in upper floor (The strength of the slab in such case will have to  be checked to allow for additional load and superimposed load)

- Where it is not feasible to provide RCC slab for ceiling as specified,  the ceiling may be fortified with Tor Steel Grills consisting of 20 mm rods spaced 3`` (75mm) centre to centre (both ways) in angle frame work.

- Where reinforcement is proposed on two faces of a R.C.O. member, it shall be staggered in such a manner that any view taken at right angles to the matt formation would show reinforcement at every 3`` centre to centre in elevation (in respect of walls) and in plan (in respect of ceiling slab)

…..2/-

-2-

- Where above reinforcement are only the   minimum and depending on the structural requirements, the structural consultants for work should design and detail out actual reinforcement required but there shall not be less than what are specified above

- All reinforcement cement concrete work for base slab, walls and roof slab shall be

of  1:2:4 mix with controlled water cement rate of 0.45. montane mixed and

compacted by vibration.

- Clear cover to maintain reinforcement not less than 25 mm..

7. All reinforcement shall be  Tor 40 or  Equivalent.

- Shuttering for Centering shall be steel  and leak proof.

- Fixing of Doors   :   While fixing the door special attention should be paid for fixing it in the plumb, as slightest bend (out of plumb line) will cause improper working of the door. Always held of the Godrej/Steelage/OEM mechanic should be made use of.

- Partition for Safe Deposit Vaults inside Strong Room :  A brick wall partition upto  8 feet height duly  plastered and oil bound distempered and grill above wall till ceiling   as approved duly grounded in the wall is to be provided.

- Electric Wiring :  It must be noted that the electric wiring should be arranged separately  for  the  strong room and it is connected  to a plug outside.  In effect when the plug is removed there should not be live electricity wiring inside the walls. roof or  floor of the strong room. Plug points at four top corners and four bottom corner are to be provided  inside the strong room apart from regular lights.

- Construction Supervision :  A certificate from our  Security Officer or registered architect / engineer has to be submitted   for construction of strong room carried as per specification given above.,